แบบเสนอหัวข้อและโครงร่างการทำโครงงานทางวิศวกรรม

ประจำภาคการศึกษาที่ 2 ปีการศึกษา 2568

- หัวข้อโครงร่าง (Thesis Title)

ภาษาไทย (ไทย) การประยุกต์ใช้จมูกอิเล็กทรอนิกส์ในการประเมินความเข้มข้นก๊าซมีเทนในนาข้าว

ภาษาอังกฤษ (English) Application of Electronic Nose for Evaluating Methane Gas Concentration in Rice Paddies

- ชื่อและรหัสนักศึกษา (Student name and code)

นาย ปัญจกุล พงษ์พานิช — รหัส 680631040 วิศวกรรมศาสตรมหาบัณฑิต วิศวกรรมเกษตร

- อาจารย์ที่ปรึกษา (Advisor)

ผศ.ดร. ณัฐวุฒิ เนียมสอน ภาควิชาวิศวกรรมเครื่องกล มหาวิทยาลัยเชียงใหม่

- บทนำ (Introduction)

  - ที่มาและความสำคัญ (Overviews)

ข้าวเป็นพืชเศรษฐกิจหลักของประเทศไทย ทั้งในด้านการส่งออกและรายได้ของเกษตรกร ในปี พ.ศ. 2567 ไทยส่งออกข้าวได้ประมาณ 10 ล้านตัน สร้างรายได้ราว 6.5 พันล้านดอลลาร์สหรัฐ [1, 2, 3] และมีครัวเรือนเกษตรกรทำนาปลูกข้าวกว่า 4.6 ล้านครัวเรือน ครอบคลุมพื้นที่มากกว่า 115,000 ตารางกิโลเมตร [1, 2] แม้สัดส่วนต่อผลิตภัณฑ์มวลรวมภายในประเทศจะไม่สูงเท่าอุตสาหกรรมอื่น ถึงอย่างนั้นข้าวยังเป็นแหล่งรายได้และมีผลต่อการดำรงชีพของครัวเรือนในชนบทจำนวนมาก ในการพัฒนาการเกษตรข้าวจึงไม่ได้จำกัดอยู่ที่การเพิ่มผลผลิตเพียงอย่างเดียว แต่รวมถึงการจัดการผลกระทบต่อสิ่งแวดล้อมที่เกิดจากรูปแบบการปลูกด้วย

การปลูกข้าวในไทยส่วนใหญ่เป็นแบบน้ำขัง ทั้งนาปีและนาปรัง [4] เมื่อมีน้ำขังในนาอย่างต่อเนื่อง สภาวะดินจะขาดออกซิเจน จุลินทรีย์กลุ่มสร้างก๊าซมีเทน (methanogens) จึงย่อยสลายอินทรียวัตถุและปล่อยก๊าซมีเทน (CH₄) ออกสู่บรรยากาศ ทั้งทางผิวน้ำและทางรากหรือลำต้นข้าว [4] ก๊าซมีเทนเป็นก๊าซเรือนกระจกที่มีศักยภาพในการกักเก็บความร้อนสูง นาข้าวจึงเป็นหนึ่งในแหล่งปล่อย CH₄ สำคัญของภาคเกษตร โดยมีรายงานว่าสัดส่วนการปล่อยจากนาข้าวอยู่ในช่วงร้อยละ 12–26 ของก๊าซเรือนกระจกจากพื้นที่เกษตร [4] ปริมาณที่ปล่อยจริงยังเปลี่ยนแปลงตามฤดูกาล การจัดการน้ำ พันธุ์ข้าว และสภาพดิน หากต้องการลดการปล่อยหรือติดตามผลของมาตรการลดก๊าซ จึงต้องอาศัยข้อมูลการวัดที่เชื่อถือได้ในระดับแปลงนา มากกว่าค่าเฉลี่ยระดับภูมิภาค

ปัจจุบันมีหลายวิธีสำหรับการวัดก๊าซมีเทนจากนาข้าว โดยแต่ละวิธีเหมาะกับบริบทการใช้งานที่ต่างกัน ตัวอย่างเช่น การใช้ห้องเก็บตัวอย่าง (static chamber) ร่วมกับก๊าซโครมาโทกราฟี (GC) ถือเป็นวิธีอ้างอิงที่ให้ความแม่นยำสูง ทั้งสำหรับการวิเคราะห์ความเข้มข้นในตัวอย่างอากาศและการประมาณอัตราการปล่อย (flux) ซึ่งวิธีที่ยอมรับกันอย่างกว้างขวาง [6, 7] แต่ต้องใช้แรงงานและต้นทุนมาก การเก็บข้อมูลทำได้ไม่บ่อย จึงติดตามการเปลี่ยนแปลงตลอดฤดูปลูกได้ยาก [6, 7] อีกวิธีหนึ่งได้แก่ เทคนิคสเปกโทรสโกปีและการสังเกตจากดาวเทียม ซึ่งให้ข้อมูลในระดับภูมิภาคหรือระดับความละเอียดในระดับนึง แต่มีต้นทุนสูงและความละเอียดเชิงพื้นที่มักไม่ระเอียดพอถึงระดับรายแปลงนา [8, 22] อีกหนึ่งทางเลือกที่ต้นทุนต่ำกว่าคือเซ็นเซอร์ก๊าซและจมูกอิเล็กทรอนิกส์ (gas sensor and electronic nose) ซึ่งเป็นการใช้เซ็นเซอร์ก๊าซใช้ร่วมกับ machine learning ในการประเมินความเข้มข้นของก๊าซ [5, 12]

แนวทาง (electronic nose, e-nose) ร่วมกับ ML มีศักยภาพสำหรับการประเมินความเข้มข้น CH₄ ในการใช้ในภาคสนาม โดยเฉพาะเมื่อนำค่าอุณหภูมิ และ ความชื้น รวมเข้าในแบบจำลองเพื่อชดเชยผลกระทบจากสภาพแวดล้อม [10] งานวิจัยก่อนหน้าจะเน้นการพิสูจน์แนวคิดหรือการทดลองในห้องแล็บ มากกว่าการประยุกต์ใช้แบบจำลอง ที่ให้ค่าความเข้มข้น CH₄ (หน่วย ppm) บนฮาร์ดแวร์ที่นำไปใช้ในแปลงจริงได้ [11, 12] และเนื่องจากข้าวไทยมีอุณหภูมิและความชื้นสูง และมีการเปลี่ยนแปลงตามฤดูกาล และสภาพแวดล้อมที่ซับซ้อน จึงยังต้องมีการพัฒนาและตรวจสอบแบบจำลองใหม่ภายใต้เงื่อนไขที่เกี่ยวข้องกับแปลงนา โดยอ้างอิงกับวิธี chamber–GC [6, 7]

วิทยานิพนธ์นี้จึงมีวัตถุประสงค์เพื่อพัฒนาและประยุกต์ใช้จมูกเล็กทรอนิกส์ในการประเมินความเข้มข้นของก๊าซมีเทนในแปลงนาข้าว

  - สรุปสาระสำคัญจากงานวิจัยที่เกี่ยวข้อง (Literature reviews)

    - การปล่อยก๊าซมีเทนจากนาข้าวน้ำขัง

      - เส้นทางปล่อยสู่บรรยากาศ

      - ปัจจัยควบคุมฟลักซ์ในระดับแปลง

      - ความแปรผันเชิงพื้นที่ภายในแปลงนา

    - การวัดก๊าซมีเทนในระดับแปลงนา

      - ห้องเก็บตัวอย่างแบบปิดร่วมกับแก๊สโครมาโทกราฟี

      - เครื่องวิเคราะห์ก๊าซความแม่นยำสูงในสนาม

      - การวัดแบบเอดดี้โคเวอเรียนซ์

    - การประเมินการปล่อยก๊าซมีเทนระดับภูมิภาค

      - การตรวจวัดจากดาวเทียม

      - แบบจำลองกระบวนการและบัญชีการปล่อย

    - การตรวจวัดก๊าซมีเทนด้วยจมูกอิเล็กทรอนิกส์และเซ็นเซอร์โลหะออกไซด์

      - การประยุกต์ในนาข้าว

      - การประยุกต์นอกแปลงนา

      - ผลของอุณหภูมิและความชื้นที่รายงานในงานทดลอง

    - การสอบเทียบความเข้มข้นก๊าซมีเทนด้วยการเรียนรู้แบบมีผู้สอน

      - แบบจำลองถดถอยเทียบค่าอ้างอิง

      - ช่วงความเข้มข้นและตัวแปรสภาพแวดล้อมที่ใช้ฝึก

  - วัตถุประสงค์ของการศึกษา (Research Objectives)

- เพื่อพัฒนาระบบจมูกอิเล็กทรอนิกส์สำหรับประเมินความเข้มข้นก๊าซมีเทนในนาข้าว

- เพื่อสร้างและตรวจสอบแบบจำลองที่ประมาณความเข้มข้นก๊าซมีเทนเป็น ppm โดยเทียบกับค่าจากเครื่อง GC ภายในขอบเขตการทดลองที่กำหนด

  - ขอบเขตของการศึกษา (Research scopes and limitations)

- ทดลองในห้องปฏิบัติการด้วยก๊าซมาตรฐานมีเทน ความเข้มข้น 5–100 ppm ที่อุณหภูมิแวดล้อม 30, 40 และ 50 องศาเซลเซียส

  - ประโยชน์ที่คาดว่าจะได้รับ (Research Contribution)

- ได้ระบบจมูกอิเล็กทรอนิกส์ที่พกพา ติดตั้งและทำงานในนาได้

- ได้ข้อมูลและแบบจำลองสำหรับประมาณความเข้มข้นก๊าซมีเทน ภายในช่วงความเข้มข้นและอุณหภูมิที่ใช้ฝึกแบบจำลอง

บทที่ 5 หลักการและทฤษฎีที่เกี่ยวข้อง

5.1 ก๊าซมีเทนในนาข้าวน้ำขัง

5.1.1 สภาวะไร้ออกซิเจนและการสร้างก๊าซมีเทนโดยจุลินทรีย์

5.1.2 ตัวแปรกระบวนการที่ทำให้ความเข้มข้นของก๊าซมีเทนเหนือผิวนา

5.1.3 วิธีการวัดก๊าซมีเทน

  - การวัดก๊าซมีเทนในแปลงขนาดเล็ก

    - กล่องเก็บตัวอย่างแบบปิด (static chamber)

    - กล่องเก็บตัวอย่างแบบพลวัต (Dynamic Flux Chamber)

    - การวิเคราะห์ก๊าซละลายในน้ำ (Dissolved Methane Analysis)

    - การติดตามด้วยไอโซโทปเสถียร (Stable Isotope Tracing)

  - การวัดก๊าซมีเทนในแปลงนาขนาดใหญ่

    - การวัดแบบเอดดี้โคเวอเรียนซ์ (eddy covariance)

    - วิธีการไล่ระดับฟลักซ์ (Flux Gradient Method)

    - สมดุลมวลสารแบบบูรณาการแนวนอน (Integrated Horizontal Flux)

  - การประเมินก๊าซมีเทนในระดับภูมิภาค

    - การรับรู้ระยะไกลผ่านดาวเทียม (Satellite Remote Sensing)

    - แบบจำลองบรรยากาศแบบย้อนกลับ (Atmospheric Inversion Models)

    - การสเกลอัปด้วยเครื่องจักรเรียนรู้ (Machine Learning Upscaling)

5.4 จมูกอิเล็กทรอนิกส์และเซ็นเซอร์ก๊าซ

5.4.1 นิยามจมูกอิเล็กทรอนิกส์: เซ็นเซอร์ก๊าซกับการรู้จำรูปแบบ

5.4.2 การทำงานของเซ็นเซอร์ก๊าซ และประเภทเซ็นเซอร์ที่เกี่ยวข้อง

5.4.3 ปัจจัยต่าง ๆ ที่ส่งผลต่อสัญญาณจากจมูกอิเล็กทรอนิกส์

5.4.4 การแปลงสัญญาณและการเก็บข้อมูล

5.5 การเรียนรู้แบบมีผู้สอน

5.5.1 ความหมายและทฤษฎีของการเรียนรู้แบบมีผู้สอน

5.5.2 การสร้างแบบจำลองในการทำนาย

  - ประเภทของแบบจำลอง

    - แบบจำลองจำแนกประเภท

    - แบบจำลองถดถอย

5.5.3 ขั้นตอนการสร้างแบบจำลองถดถอย

  - การจัดการข้อมูล

  - การแบ่งชุดข้อมูล

  - การเลือกโมเดลและอัลกอริทึม

  - การประเมินประสิทธิภาพโมเดล

5.5.4 การปรับแต่งโมเดลและการนำไปใช้งาน ตัวชี้วัดความใกล้เคียงกับค่าอ้างอิง: RMSE, MAE และ R²

บทที่ 6 วิธีดำเนินการวิจัย

6.1 ขอบเขตการทดลองและตัวแปร

- 6.1.1 สถานที่และลักษณะการทดลองในห้องปฏิบัติการ

- 6.1.2 ตัวแปรอิสระ: ระดับความเข้มข้น CH₄ และอุณหภูมิ

- 6.1.3 ตัวแปรตาม: ความเข้มข้น CH₄ จาก GC

6.2 วัสดุและอุปกรณ์

- 6.2.1 ระบบจมูกอิเล็กทรอนิกส์

  - การไหลของกลิ่น

  - เซ็นเซอร์ก๊าซ

  - เซ็นเซอร์อุณหภูมิและความชื้น

  - หน่วยควบคุม

- 6.2.2 ห้องสภาพแวดล้อมจำลอง

- 6.2.3 ก๊าซมาตรฐาน

6.3 การทดลองและเก็บข้อมูล

- 6.3.1 การเตรียมความเข้มข้นและการตั้งอุณหภูมิตาม 6.1

- 6.3.2 การบันทึกสัญญาณ (ช่วงอากาศอ้างอิงและช่วงวัด) พร้อมเก็บตัวอย่างอากาศส่ง GC

6.4 การวิเคราะห์ก๊าซมีเทนด้วย GC

6.5 การสร้างและประเมินแบบจำลอง

- 6.5.1 การจัดการข้อมูล

- 6.5.2 การฝึกแบบจำลองเพื่อทำนายความเข้มข้นของก๊าซมีเทน ด้วยการเรียนรู้แบบมีผู้สอนชนิดถดถอย

- 6.5.3 การประเมินแบบจำลอง

6.6 การทดสอบในแปลงนา

6.6.1 การติดตั้งกล่องเก็บตัวอย่างแบบปิดและการติดตั้งระบบจมูกอิเล็กทรอนิกส์

6.6.2 การวัดและเก็บข้อมูล

6.6.3 การวิเคราะห์ข้อมูลจากการทดสอบในแปลงนา

เอกสารอ้างอิง (References)

[1] C. Sowcharoensuk, "Industry Outlook 2026-2028: Rice Industry," Krungsri Research, 2026. [Online]. Available: https://www.krungsri.com/th/research/industry/industry-outlook/agriculture/rice/io/io-rice-2026-2028

[2] T. Nation, "Thai farm incomes face pressure from global rice market shifts," The Nation, 2026.

[3] Thairath, "Thai Rice Exports in 2025 Surpass Targets Efforts Accelerate in 2026 to Maintain and Expand Markets," Thairath, 2026.

[4] H. Nguyen et al., "Carbon Footprint Reduction from Closing Rice Yield Gaps," in Carbon Footprint of Rice Production, 2023, pp. 149–176.

[5] P. Rajasekar and J. A. V. Selvi, "Sensing and Analysis of Greenhouse Gas Emissions from Rice Fields to the Near Field Atmosphere," Sensors, vol. 22, no. 11, p. 4141, 2022.

[6] M. Zaman et al., "Methodology for measuring greenhouse gas emissions from agricultural soils using non-isotopic techniques," in Measuring Emission of Agricultural Greenhouse Gases and Developing Mitigation Options Using Nuclear and Related Techniques, Springer, 2021, pp. 11–108.

[7] N. J. Mumu et al., "Methodological progress in the measurement of agricultural greenhouse gases," Carbon Management, vol. 15, no. 1, p. 2366527, 2024.

[8] L. Tyagi et al., "Environmental impacts and recent advancements in the sensing of methane: a review," Environ. Technol. Rev., vol. 14, no. 1, pp. 191–212, 2025.

[9] M. S. Borhan and M. M. Khanaum, "Sensors and methods for measuring greenhouse gas emissions from different components of livestock production facilities," J. Geosci. Environ. Prot., vol. 10, no. 12, pp. 242–272, 2022.

[10] G. Domènech-Gil et al., "Electronic nose for improved environmental methane monitoring," Environ. Sci. Technol., vol. 58, no. 1, pp. 352–361, 2023.

[11] S. Baruah and D. H. Mazumder, "A Review on Application of Machine Learning Techniques Coupled With E-Nose in Healthcare Agriculture and Allied Domains," IEEE Access, 2025.

[12] A. Ahmad et al., "The Promise of Low-Cost Metal-Oxide Semiconductor Gas Sensors for Precision Agriculture," Adv. Sensor Res., 2026.

[13] H. Zhou et al., "Paddy rice methane emissions, controlling factors, and mitigation potentials across Monsoon Asia," Sci. Total Environ., vol. 935, p. 173441, 2024, doi: 10.1016/j.scitotenv.2024.173441.

[14] T. D. Xuan, T. T. N. Minh, R. Rayee, N. Dong, and N. X. Chien, "Advances in mitigating methane emissions from rice cultivation: past, present, and future strategies," Environ. Sci. Pollut. Res., 2025, doi: 10.1007/s11356-025-36776-8.

[15] A. Rafy, M. Hannan, M. Mohammed, and N. Khan, "Meta-Analysis of Alternate Wetting and Drying (AWD) Irrigation Effects on Methane and Nitrous Oxide Emissions Across Different Climates and Soil Types," Eur. J. Ecol. Biol. Agric., vol. 2, no. 5, pp. 181-200, 2025, doi: 10.59324/ejeba.2025.2(5).13.

[16] S. S. Anapalli, S. R. Pinnamaneni, K. N. Reddy, P. Wagle, and A. J. Ashworth, "Eddy covariance assessment of alternate wetting and drying floodwater management on rice methane emissions," Heliyon, vol. 9, no. 4, p. e14696, 2023, doi: 10.1016/j.heliyon.2023.e14696.

[17] S. Guan, Z. Qi, S. Li, S. Du, and D. Xu, "Effects of rice root development and rhizosphere soil on methane emission in paddy fields," Plants, vol. 13, no. 22, p. 3223, 2024, doi: 10.3390/plants13223223.

[18] D. Yusong et al., "Methane emissions from rice paddies are regulated by carbon availability and soil pH along a mean annual temperature gradient," Sci. Rep., vol. 16, p. 14129, 2026, doi: 10.1038/s41598-026-43940-8.

[19] T. Yang, M. Wang, X. Wang, C. Xu, F. Fang, and F. Li, "Product type, rice variety, and agronomic measures determined the efficacy of enhanced-efficiency nitrogen fertilizer on the CH4 emission and rice yields in paddy fields: A meta-analysis," Agronomy, vol. 12, no. 10, p. 2240, 2022, doi: 10.3390/agronomy12102240.

[20] Q. Hu et al., "Global methane emissions from rice paddies: CH4MOD model development and application," iScience, vol. 27, no. 11, p. 111237, 2024, doi: 10.1016/j.isci.2024.111237.

[21] T. B. T. Vo et al., "Measurement approaches for greenhouse gas emissions from rice II: advanced technology for accelerating throughput," Front. Agron., vol. 7, p. 1693620, 2026, doi: 10.3389/fagro.2025.1693620.

[22] Y. Xu, A. Yazdinejad, H. Wang, and J. D. Kong, "From detection to decision: A systematic literature review of AI and machine learning evolution in methane modelling," SSRN, 2025, doi: 10.2139/ssrn.5218753.

[23] Q. Zhang et al., "Machine learning-driven method for in-situ high-frequency CH4 measurement in paddy fields based on water-soil-air factors," J. Environ. Manage., vol. 393, p. 127132, 2025, doi: 10.1016/j.jenvman.2025.127132.

[24] Z. Ye, Y. Liu, and Q. Li, "Recent progress in smart electronic nose technologies enabled with machine learning methods," Sensors, vol. 21, no. 22, p. 7620, 2021, doi: 10.3390/s21227620.

[25] J. Yin et al., "Rapid identification method for CH4/CO/CH4-CO gas mixtures based on electronic nose," Sensors, vol. 23, no. 6, p. 2975, 2023, doi: 10.3390/s23062975.

[26] L. Fu, S. You, G. Li, X. Li, and Z. Fan, "Application of semiconductor metal oxide in chemiresistive methane gas sensor: Recent developments and future perspectives," Molecules, vol. 28, no. 18, p. 6710, 2023, doi: 10.3390/molecules28186710.

[27] B. Andrews, A. Chakrabarti, M. Dauphin, and A. Speck, "Application of machine learning for calibrating gas sensors for methane emissions monitoring," Sensors, vol. 23, no. 24, p. 9898, 2023, doi: 10.3390/s23249898.

[28] H. L. Mitchell, S. J. Cox, and H. G. Lewis, "Calibration of a low-cost methane sensor using machine learning," Sensors, vol. 24, no. 4, p. 1066, 2024, doi: 10.3390/s24041066.

[29] J. W. Creswell, *Research Design: Qualitative, Quantitative, and Mixed Methods Approaches*, 3rd ed. SAGE Publications, 2009.

[30] B. Madsen, *Statistics for Non-Statisticians*. Springer-Verlag Berlin Heidelberg, 2011. DOI: 10.1007/978-3-642-17656-2

[31] M. P. Deisenroth, A. A. Faisal, and C. S. Ong, *Mathematics for Machine Learning*. Cambridge University Press, 2020.

[32] M. Saunois et al., "Global Methane Budget 2000–2020," Earth Syst. Sci. Data, vol. 17, no. 5, pp. 1873–1958, 2025, doi: 10.5194/essd-17-1873-2025.

[33] IPCC, "Agriculture, Forestry and Other Land Uses (AFOLU)," in Climate Change 2022: Mitigation of Climate Change. Cambridge, U.K.: Cambridge Univ. Press, 2023, pp. 747–860, doi: 10.1017/9781009157926.009.

[34] R. Conrad, "Methane production in soil environments—Anaerobic biogeochemistry and microbial life between flooding and desiccation," Microorganisms, vol. 8, no. 6, p. 881, 2020, doi: 10.3390/microorganisms8060881.

[35] A. Z. Oo, K. T. Win, and S. D. Bellingrath-Kimura, "Within field spatial variation in methane emissions from lowland rice in Myanmar," SpringerPlus, vol. 4, p. 145, 2015, doi: 10.1186/s40064-015-0901-2.

[36] K. Minamikawa, T. Tokida, S. Sudo, A. Padre, and K. Yagi, Guidelines for Measuring CH4 and N2O Emissions from Rice Paddies by a Manually Operated Closed Chamber Method. Tsukuba, Japan: NIAES, 2015.

[37] IPCC, "Cropland," in 2019 Refinement to the 2006 IPCC Guidelines for National Greenhouse Gas Inventories, vol. 4. IPCC, 2019, ch. 5.

[38] R. Wassmann et al., "Increasing sensitivity of methane emission measurements in rice through deployment of closed chambers at nighttime," PLOS ONE, vol. 13, no. 2, p. e0191352, 2018, doi: 10.1371/journal.pone.0191352.

[39] T. E. R. Mazengo, X. Zhong, X. Liu, M. F. Mwema, and R. Gill, "Non-flow-through static (closed chamber) method for sampling of greenhouse gases in crop production systems," Front. Agron., vol. 6, p. 1464495, 2024, doi: 10.3389/fagro.2024.1464495.

[40] T. B. T. Vo, R. Wassmann, B. O. Sander, and A. M. Radanielson, "Measurement approaches for greenhouse gas emissions from rice I: technical evolution and scientific results obtained with different methods," Front. Agron., vol. 8, p. 1693619, 2026, doi: 10.3389/fagro.2026.1693619.

[41] G. Zhang et al., "Fingerprint of rice paddies in spatial–temporal dynamics of atmospheric methane concentration in monsoon Asia," Nat. Commun., vol. 11, p. 554, 2020, doi: 10.1038/s41467-019-14155-5.

[42] R. Liang et al., "Satellite-based monitoring of methane emissions from China’s rice hub," Environ. Sci. Technol., vol. 58, no. 52, pp. 23127–23137, 2024, doi: 10.1021/acs.est.4c09822.

[43] W. Eugster and G. W. Kling, "Performance of a low-cost methane sensor for ambient concentration measurements in preliminary studies," Atmos. Meas. Tech., vol. 5, no. 8, pp. 1925–1934, 2012, doi: 10.5194/amt-5-1925-2012.

[44] C. J. Jørgensen, J. Mønster, K. Fuglsang, and J. R. Christiansen, "Continuous methane concentration measurements at the Greenland ice sheet–atmosphere interface using a low-cost, low-power metal oxide sensor system," Atmos. Meas. Tech., vol. 13, no. 6, pp. 3319–3328, 2020, doi: 10.5194/amt-13-3319-2020.

[45] A. Collier-Oxandale et al., "Assessing a low-cost methane sensor quantification system for use in complex rural and urban environments," Atmos. Meas. Tech., vol. 11, no. 6, pp. 3569–3594, 2018, doi: 10.5194/amt-11-3569-2018.

[46] R. Rivera Martinez et al., "The potential of low-cost tin-oxide sensors combined with machine learning for estimating atmospheric CH4 variations around background concentration," Atmosphere, vol. 12, no. 1, p. 107, 2021, doi: 10.3390/atmos12010107.

[47] T. Tokida, "Increasing measurement throughput of methane emission from rice paddies with a modified closed-chamber method," J. Agric. Meteorol., vol. 77, no. 2, pp. 160–165, 2021, doi: 10.2480/agrmet.d-20-00029.

[48] C. Li et al., "Low-cost detection of methane gas in rice cultivation by gas chromatography-flame ionization detector based on manual injection and split pattern," Molecules, vol. 27, no. 13, p. 3968, 2022, doi: 10.3390/molecules27133968.

[49] D. Bastviken, J. Nygren, J. Schenk, R. Parellada Massana, and N. T. Duc, "Technical note: Facilitating the use of low-cost methane (CH4) sensors in flux chambers," Biogeosciences, vol. 17, no. 13, pp. 3659–3667, 2020, doi: 10.5194/bg-17-3659-2020.

[50] E. Kiplimo et al., "Addressing low-cost methane sensor calibration shortcomings with machine learning," Atmosphere, vol. 15, no. 11, p. 1313, 2024, doi: 10.3390/atmos15111313.

[51] R. Lakhmi et al., "Linear and non-linear modelling methods for a gas sensor array developed for process control applications," Sensors, vol. 24, no. 11, p. 3499, 2024, doi: 10.3390/s24113499.

[52] A. Shah et al., "Characterising the methane gas and environmental response of the Figaro Taguchi Gas Sensor (TGS) 2611-E00," Atmos. Meas. Tech., vol. 16, no. 13, pp. 3391–3419, 2023, doi: 10.5194/amt-16-3391-2023.

[53] D. Furuta, T. Sayahi, J. Li, B. Wilson, A. A. Presto, and J. Li, "Characterization of inexpensive metal oxide sensor performance for trace methane detection," Atmos. Meas. Tech., vol. 15, no. 17, pp. 5117–5128, 2022, doi: 10.5194/amt-15-5117-2022.

[54] Z. Chen et al., "Global Rice Paddy Inventory (GRPI): A high-resolution inventory of methane emissions from rice agriculture based on Landsat satellite inundation data," Earth’s Future, vol. 13, no. 4, 2025, doi: 10.1029/2024EF005479.

[55] I. Nouchi, S. Mariko, and K. Aoki, "Mechanism of methane transport from the rhizosphere to the atmosphere through rice plants," Plant Physiol., vol. 94, no. 1, pp. 59–66, 1990, doi: 10.1104/pp.94.1.59.

[56] M. M. Othman et al., "A Robust Support Vector Machine Model for Monitoring Methane Emission Levels in Paddy Ecosystems using Electronic Nose Technology," J. Adv. Res. Appl. Sci. Eng. Technol., vol. 60, no. 5, pp. 33–50, 2026, doi: 10.37934/araset.60.5.3350.